| Total Amount (PKR) | Specifications | S# |
| --- | --- | --- |
| : | Provision and installation of AC submersible pump 0.75 HP (DAYUAN or equivalent) with 1.0" HDPE suction pipe (P-100 DIN 8074 & 8075) along with all fittings and accessories like AC wire 70ft avg. conduit pipes, plastic coated steel cable 7mm for hanging pipe and pump, pump clips of stainless steel. Complete in all respect | 1 |
|  | Polycrystalline Solar Panel Tier 1. Minimum open circuit voltage of single panel = 45V. Minimum and maximum Power capacity range of single Panel = 330 watts to 450 watts. Minimum total capacity and required 4 or 6 panels Proposed panels should provide sufficient watts ! volts to operate Pump inverter mentioned in RFQ Panel Manufacturer options are as under (Jinko, Canadian Solar, Astronergy, Longi or equivalent but only original panels are acceptable, tampered and counterfeit panels are not acceptable in any case) Delivered Panels should pass the authenticity by the respective manufacturer | 2 |
|  | 10 SWG Galvanize Frame with rust free nuts/bolts, frame quantity should be as per proposed panels per site. Reference layout of frame is attached for quidance | 3 |
|  | Pump Inverter (INVT or equivalent), 2.2 KW, Single phase - Compatible with proposed panels / circuit. | 4 |
|  | Breakers and switches | 5 |
|  | Provision and installation of inverter and breaker box. Complete in all respect | 6 |
|  | Providing and laying PCC 1:2:4 foundation pad for fixing solar panel frames (1 foot raised from ground and should be in same level) including anchoring bolts; complete in all respect. Installation of solar system including conduit pipes, connectors, solution taps, etc. Complete in all respect Set | 7 |
|  | All panels of a site should be in series circuit and this string / circuit connected to inverter with required safety requirements. Main DC line should be under ground and protected by PVC pipe. Reference Layout of panels is attached for guidance | 8 |
|  | Tool kit for community (pliers, screwdriver, solution tape, electrical tester pen, panel nut bolt key, etc.) | 9 |
|  | 1.Tank Size 7 x 9 Height 8ft with 2ft RCC Bad 2.RCC pillar 3. Bricks 4. Roof 5. Animal water Drink pond size 7 x 3 6. Four tabs 7. VFT Box | 10 |
|  | 1.Donor name plaque size 2 x 2 pure Wight ziarat marble 2.Standee banner with stand 3. Reporting 6 times 4.color weather sheet | 11 |
|  | Boundary wall height 4 feet with Iron gate 2.5x3 | 12 |
|  | Total amount in words | 11 |

Proce Schedule: